Experiment Number: 3

MATLAB Analysis of Spectrum Bands, Propagation Loss and Cellular Interference

Aim

To compare a simplified free-space path-loss estimate across representative sub-1 GHz, mid-

band and mm Wave frequencies, and examine how distance and frequency reuse affect

received signal and a simple interference measure.

Objectives

By the end of the experiment, students should be able to:

1. Calculate or simulate path loss at representative sub-1 GHz, mid-band and mmWave

frequencies.

2. Compare estimated received signal strength as frequency and distance change.

3. Examine how cell separation or frequency reuse changes a simplified interference ratio.

4. Relate the results to coverage and spectrum-use considerations across mobile generations,

while stating model limitations.

Requirements

• Computer with MATLAB R2019b or later.

• No RF hardware is required.

Theoretical Background

- The free-space path-loss equation in decibels is FSPL = 32.44 + 20log10(f_MHz) +20log10(d_km), where f is carrier frequency and d is separation.

- With fixed transmit powerand antenna gains, estimated received power is P_r = P_t + G_t + G_r − FSPL.

- In thisidealized equation, doubling distance adds about 6 dB of loss and doubling frequency addsabout 6 dB of loss at the same distance.

- For a simple reuse illustration, let the desired transmitter be at distance R and six equal-powerco-channel interferers be at distance D = √(3N)R, where N is a reuse-cluster size.

- Under a power-law path-loss exponent n, approximate SIR = (D/R)^n / 6. This idealized first-tier model omits shadowing, fading, sector antennas, unequal loads, terrain, adjacent-channel leakage and real frequency-planning constraints. Because desired and interfering links use the same frequency, frequency-dependent free-space loss cancels in this simplified SIR ratio.

Parameters and Assumptions

1. Carrier Bands: 0.7 GHz, 3.5 GHz, and 28 GHz — Representative sub-1 GHz, mid-band, and millimetre-wave frequency points.

2. Transmit Power: 30 dBm — Illustrative conducted transmit power, with antenna gains assumed to be 0 dBi.

3. Distance Range: 0.1 km to 2 km — The range used for free-space propagation comparisons.

4. Path-Loss Exponent for Reuse Model: 3.5 — An illustrative non-free-space distance exponent used in the frequency-reuse model.

5. Reuse Cluster Sizes (N): 1, 3, 7, and 12 — Different cluster sizes used to compare frequency reuse; larger \(N\) implies greater idealized co-channel separation.

Procedure

1. Run the script and inspect the path-loss and received-power plots.

2. Record the estimated received power at 0.5 km for each frequency.

3. Inspect the reuse-SIR plot and compare the baseline N values.

4. Change transmit power, distance range, path-loss exponent or reuse sizes; rerun and

record the effect.

5. Explain why frequency reuse changes the simplified co-channel SIR and why the free-

space link budget is not a complete coverage prediction.

MATLAB Program — Path Loss, Received Power and Reuse Interference

% Experiment 3: illustrative propagation and frequency reuse

clear; clc; close all;

% Representative bands and link-budget assumptions

freqGHz = [0.7 3.5 28];

freqMHz = freqGHz * 1000;

bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"];

txPowerdBm = 30;

gtDbi = 0; grDbi = 0;

distanceKm = logspace(log10(0.1),log10(2),200);

distanceAtPointKm = 0.5;

% Free-space path loss (d in km, f in MHz)

fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm);

lossAtPoint = fspl(freqMHz,distanceAtPointKm);

rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

linkTable = table(bandName,freqGHz(:),lossAtPoint(:),rxAtPoint(:), ...

'VariableNames',{'Band','Frequency_GHz','FSPL_dB','EstimatedRx_dBm'});

disp(linkTable);

% Path loss and received power over distance

figure; tiledlayout(1,2);

nexttile; hold on; grid on;

for k=1:numel(freqMHz)

plot(distanceKm,fspl(freqMHz(k),distanceKm),'LineWidth',1.5);

end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Path loss (dB)');

legend(bandName,'Location','northwest'); title('Free-Space Path Loss');

nexttile; hold on; grid on;

for k=1:numel(freqMHz)

rx = txPowerdBm + gtDbi + grDbi - fspl(freqMHz(k),distanceKm);

plot(distanceKm,rx,'LineWidth',1.5);

end

set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Estimated received power (dBm)');

legend(bandName,'Location','southwest'); title('Illustrative Received Power');

% Simple first-tier co-channel reuse estimate

cellRadiusKm = 0.5;

reuseN = [1 3 7 12];

pathLossExponent = 3.5;

reuseDistance = sqrt(3*reuseN) * cellRadiusKm;

sirLinear = (reuseDistance/cellRadiusKm).^pathLossExponent / 6;

sirDb = 10*log10(sirLinear);

reuseTable = table(reuseN(:),reuseDistance(:),sirDb(:), ...

'VariableNames',{'ReuseClusterN','ApproxInterfererDistance_km','ApproxSIR_dB'});

disp(reuseTable);

figure; plot(reuseN,sirDb,'o-','LineWidth',1.5); grid on;

xlabel('Reuse cluster size N'); ylabel('Approximate SIR (dB)');

title('Simplified Co-Channel SIR versus Reuse Cluster Size');

fprintf('At %.2f km, estimated received powers are:\n',distanceAtPointKm);

disp(table(bandName,rxAtPoint(:),'VariableNames',{'Band','Rx_dBm'}));

Result

Conclusion

Summarize the frequency and distance trends from the simulation and the effect of reuse separation on the illustrative SIR. Identify the free-space assumption, fixed transmit power and idealized six-interferer geometry as limitations. Note that practical coverage depends on antenna characteristics, environment, bandwidth, receiver sensitivity, interference, regulatory constraints.